FM 900 Field Personality Programmer (FPP) Guide

(applicable to FPP Version 3.0 - the altered version)

HARDWARE CODE MADE UP OF EIGHT DIGITS:   (ie: ABCDEFGH)

Digit:

A - always 9 for FM900 mobiles

B - 1=High simplex (FM91), 2=Medium simplex (FM92), 3=Low simplex (FM93), 7=FM92 with FM91 PCBs (FM97)

C - Band		2= E band (66-88 MHz),	3= B band (132-153 MHz)

4= A band (148-174 MHz),	5= T band (403-440 MHz)

6= U band (440-470 MHz),	7= W1 band (470-500 MHz)

8= W2 band (500-520 MHz)

These are the suggested TX limits, however, RX limits

can be programmed upto 30 MHz either side of the specified

band edge.  No guarantee of PLL lock, however.

D - Radio type.   	3= local control, 25 KHz chnl spacing

4= remote control, 25 KHz chnl spacing

7= local control, 12.5 KHz chnl spacing

8= remote control, 12.5 KHz chnl spacing

E - Mechanical construction

0= standard

1= extended temperature, 5ppm (crystal heater)

2= waterproof

3= extended temp & waterproof.

5= extended temp. TXCO

7= ext. temp range, waterproof TXCO

(when using a waterproof radio, a waterproof option in the hardware

code must be used.)

F - Signalling hardware

0= none

1= CTCSS encode only

2= CTCSS enc/decode

3= No CTCSS,  Selcall type 1 (G =>1) (Selcall type 1=encode only)

4= CTCSS encode, Selcall type 1

5= CTCSS enc/dec, Selcall type 1

6= No CTCSS, Selcall type 2 (type 2= encode/decode selcall)

7= CTCSS encode, Selcall type 2

8= CTCSS enc/dec, Selcall type 2

Note on Selcall encode & decode: Selcall type 1 can both send selcalls

and respond to only one received selcall code and group call if applicable.

Selcall type 2 can both send selcalls and respond to and receive multiple

and variable selcalls (used for sending status, etc)

G - Selcall options

0= none

1= CCIR selcall

2= ZVE 1 selcall

3= EEA selcall

4= EIA selcall

5= No selcall with I/O expansion kit

6= CCIR selcall, encode type 2

7= ZVE 1 selcall, encode type 2

8= EEA selcall, encode type 2

9= EIA selcall, encode type 2

Note that most selcall hybrids can only respond to CCIR selcall tones,

the other selcall modes were not well supported.

H - Other options

0= standard version

1= reverse tone burst

3= dual front end

4= ignition noise blanker

SOFTWARE CODE MADE UP OF 15 DIGITS

Digit:

A - Channel Display

0= intermittent display

1= continuos display

2= intermittent + 8 hour timer (auto power off)

3= continuos display + 8 hour timer

B - Mute

0= fixed, with defeat

1= variable with defeat

2= fixed, no defeat

3= variable, no defeat

C - Economiser options

0= off

1= economiser on all channels

2= operator selectable

3= economiser on a per channel basis

D - TX timer

0= off

1= 1 minute

2= 3 minutes

3= 5 minutes

4= 10 minutes

E - TX inhibit	0= off		1= TX inhibit all channels

F - TX power

0= 25 watts

1= 5 watts

2= 1 watt

3= operator selectable

4= per channel option

6= 5-25 watts

7= 1-25 watts adjustable

G - Scanning

0= none

1= scan vacant pre programmed blocks

2= scan occupied pre programmed blocks

3= scan occupied, operator selectable, pre programmed blocks

H - Voting	0= no voting, 	1= voting enabled

I - CTCSS options

0= no CTCSS

1= CTCSS variable per channel

2= variable via keypad

3= variable per channel, RTB (reverse tone burst)

4= variable via keypad, RTB

5= Community Repeater (CR) per channel

6= via keypad and CR

7= CR per channel and RTB

J - Tone period

0= no tone period

1= 100 ms

2= 40 ms

3= 70 ms

4= 20 ms

5= 33 ms

K - Simplex mode 	0= simplex mode (no other option selectable)

LL - Selcall encode (only selectable if selcall is fitted, otherwise program 00)

Selections available: 00-05, 07-09, 11, 12, 15, 16 ,27 ,28, 35 ,36 ,43 ,44 ,51-53

MM - Selcall decode (only selectable if selcall is fitted, otherwise program 00)

Selections available:	00-18 for selcall type 2

00, 31-46, 50, 51 for selcall type 1

Note on Selcall software selection codes: selection of all codes is not available to all radios.  See selcall selection chart for the type of radio you have. (not included here due to large size & complexity)

Common error messages:

*Selecting CTCSS tones without the correct hardware being installed

at the hardware code and the like will cause error messages.  This

applies to things like selcall setups etc.

*TX inhibit cannot be selected with Community repeater

*Tone periods 33 & 20 ms are incompatible with type 2 selcall.

*Voting cannot be selected with mute defeat.

*Selcall decode (in software code) must =00 if encode =03 or =07

*Selcall encode 04 & 08 with decode 00 is an invalid combination if

type 2 selcall is fitted

*Selcall software code 0400 & 0800 can still be used for special

applications (no decode, ident with PTT, and base selcall encode when

operating ÔsendÕ button)

*Community repeater must have CTCSS encode/decode hardware

available, and requires CTCSS tone to be entered.

*Voting isn't selectable without NSC810 I/O expander hardware

*Can only have duplicate channel numbers with voting selected on

those duplicate channel numbers

*Cannot select economiser and voting on same channel

*Cannot select economiser and selcall on same channel

*Cannot select selcall and scanning on same channel

*Must have more than one channel in a vote group

Programming notes:

NEVER program in XXXXXXX for TX or RX frequency (VCO goes

unlocked no matter what)

NEVER program in 000.000 for TX or RX frequency (TX will be

very spurious  no matter what)

IF you want to create a channel as RX only, program in a TX frequency

that you are licensed for or a UHF CB channel if in U, W1 or W2 band.

BE VERY CAREFUL when programming in frequencies - there

should only be 4 digits after the decimal point and no spaces to pad out

the last position, and no leading zeros in the digits before the decimal

point in the case of E band VHF.

Incorrect programming will have one of three effects:

radio will not power up (most common)

radio will have unlocked VCO on TX or RX or both (radio inoperable)

radio will cause severe interference due to the transmission of spurii.

3 scan groups available for FM91, FM97

2 scan groups available for FM92, FM93 and all waterproof radios

10 channels per scan group, with one of these only available as a

priority channel

Channel number range for FM91, FM97 is 1-120; FM92 is 1-99;

FM93 is 1-10

Power selection entry is by the letters H, M, L & S (for high, medium,

low and selectable) and must be consistent with the type of radio.

CTCSS Tone selection table

67.0	Q	88.5	A	114.8	D	146.2	P	186.2	3	241.8	*

71.9	R	91.5	W	118.8	M	151.4	H	192.8	4	250.3	#

74.4	S	94.8	J	123.0	E	156.7	X	203.5	5	97.4	=

77.0	T	100.0	B	127.3	N	162.2	Y	210.7	6

79.7	U	103.5	K	131.8	F	167.9	0	218.1	7

82.5	I	107.2	C	136.5	0	173.1	1	225.7	8

85.4	V	110.9	L	141.3	G	179.9	2	233.6	9

Community repeater: select tone Z

Notes on FPP Version 3.0 (altered)

Normally, FPP will restrict programming of the radio in two ways: first, transmission and reception was fixed within the one band ie. If you had a U band radio 450 to 470 MHz, you could not program in any frequency for outside this band.  In this altered version of FPP however, this restriction is lifted.  Bear in mind that the VCO has its limits, however.  Depending on your setup and tuning, upto 20 MHz reception outside of the band your radio was designed for may be possible.  The second restriction was to only allow the selection of low or medium transmit power (1 or 5 Watts) on UHF CB.  User selectable power levels was not an option.  In this altered version of FPP, this restriction is also lifted, and as a by-product will allow high power selection for UHF CB as well.  This altered version of FPP does have its bugs, too.  If you try to program in a RX only channel by programming in xxxxxx or 000.0000 as the TX frequency, the radio will either not operate at all, go VCO unlocked on RX or transmit large amounts of spurious signals.  BE WARNED!

Help! I get an error message saying the EPROM programmer is not initialised!

That is OK.  FPP was designed to work with an internal EPROM programming card specific to FPP.  What you will need to do is to take the file that FPP saves to disk, and write that file as it is to an EPROM.  If you create a personality called, say, “myfreq”, you will find a file called myfreq.bin in the directory x:\fpp\fm900\jobs\myfreq\    Take this file, and copy it to your root directory and then using whatever EPROM burning equipment/software you have, write this file to the EPROM.

Notes for Programming EPROMS using Electronics Australia kit (Dick Smith kit) with Eprom.exe software:

Eprom types suitable for use: 2764 (21-25 V pgm), 2764A (12.5 V pgm) or other 27x64-x EPROMs.

Set device switch on programmer

Set Vcc = 5V

Connect programmer, insert EPROM and turn on.

Select:

Vpp = 12.5V, write pulse 1ms, 1 pulse per byte, final pulse = 0, EPROM multi pulse.  If this does not work, then try:

Vpp = 12.5V, write pulse 10ms, 1 pulse per byte, final pulse = 0, standard EPROM single pulse.  If this does not work, then try:

Vpp = 12.5V, write pulse 50ms, 1 pulse per byte, final pulse = 0, standard EPROM single pulse.  If this does not work, then try:

Vpp = 21V, write pulse 1ms, 20 pulses per byte, final pulse = 0, EPROM multi pulse.  If this does not work, then try:

Vpp = 21V, write pulse 50ms, 1 pulse per byte, final pulse = 0, standard EPROM single pulse.  If this does not work, then try:

Vpp = 25V, write pulse 50ms, 1 pulse per byte, final pulse = 0, standard EPROM single pulse.

When comparing binary files using DOS command ÔFCÕ, donÕt forget the binary code output from FPP contains the date; two identical personalities may not match using FC due to the different date encoded into the binary file, at locations 1FE2, 1FE3 & 1FE4 from the start of the file, and the checksum, see note below.

Location 1F64 appears to be a data checksum for the whole data stored on the EPROM.